Figure 1: Program Initialization and Start of the 'CONV_TRACK_FULL' Loop.

Figure 2: Complete Defined Position Coordinates and Variables Data Block.

| Expt No: 06 | AUTONOMOUS MATERIAL HANDLING AND MANIPULATION LOGIC |
| --- | --- |
| Date: 12 / 03 /2026 |  |

- AIM:

To design and implement an automated pick-and-place robotic sequence using asynchronous sensor handshaking, effector actuation, and precise spatial trajectory mapping on a 6-axis industrial manipulator.

- APPARATUS REQUIRED:

  - Robotic System: Mitsubishi RV-8CRL-D industrial manipulator.

  - Control Hardware: CR800-Series Controller.

  - Software Suite: RT ToolBox3 integrated engineering environment.

  - Sensing Hardware: Photoelectric workpiece detection sensor mapped to Input 6031.

  - Actuators: Component transport conveyor and pneumatic suction/gripper assembly actuated via Outputs 12/13.

- THEORY:

A fundamental robotic pick-and-place operation requires the seamless integration of spatial motion planning with digital logic control to interact with the external environment.

- Core Principle:

Autonomous manipulation relies on event-driven execution. The robot remains in a standby state until an external physical sensor triggers the initiation of a predefined sequence of kinematic sub-routines (retrieval, transfer, and placement).

- Working Mechanism:

  - Approach/Escape Kinematics: To prevent lateral sheer collisions with fixtures or components during retrieval and placement, the kinematic sequence strictly enforces relative Z-axis offsets (approach and escape heights). This ensures the tool only enters the manipulation zone vertically.

  - I/O Synchronization: The program architecture utilizes Wait M_In logic to halt motion execution until workpiece presence is verified by external instrumentation. Subsequent M_Out signals are triggered to electronically actuate pneumatic valves for payload securing and release.

  - Point-to-Point Transfer: The sequence utilizes a combination of joint interpolation (Mov) for rapid spatial repositioning and linear interpolation (Mvs) for the precise vertical insertion into the pick and place coordinates.

Figure 3: Maximized 3D Simulation Pane with Real-Time Coordinate Overlay at 'PGET' (Conveyor) Pick Station.

- PROCEDURE:

  - Setup & Connections:

Interface the photoelectric sensor and pneumatic solenoid valves with the controller's I/O terminals. Configure the conveyor system and define the absolute spatial coordinates for the "Pick" (pPick) and "Place" (pPlace) stations.

  - Algorithm & Logic:

    - Activate the component conveyor and enter a continuous monitor loop for workpiece arrival.

    - Upon photoelectric detection, halt the conveyor to stabilize the workpiece.

    - Execute the kinematic "Pick" sequence: move to a safe Z-offset above the part, perform a linear approach, engage the pneumatic effector, and execute a vertical escape.

    - Execute the "Place" sequence: move to a safe Z-offset above the drop zone, perform a linear insertion, disengage the pneumatic effector, and execute a vertical escape.

    - Reactivate the conveyor and loop the process for continuous automation.

  - Execution Steps:

    - Teach and record the absolute coordinates for pPick and pPlace using the teaching pendant.

    - Verify the "Hand Type" parameter in RT ToolBox3 matches the installed pneumatic assembly logic.

    - Validate the digital I/O handshake mechanism utilizing the Signal Monitor screen.

    - Execute a Step Run to ensure the Z-offsets safely clear all environmental obstacles before switching to Automatic mode for full-speed execution.

  - Observation Instructions:

Analyze the operational Tact Time (cycle time from pick to place) and monitor the pneumatic grip stability during rapid joint interpolation (Mov) between the pick and place stations.

- RESULT AND OBSERVATION:

The robotic system successfully performed synchronized pick-and-place operations with external conveyor and sensor systems. Proper approach/escape height control prevented interference, and reliable sensor handshaking ensured accurate part handling, as verified by faculty.