## Figure 1: Servo Motor.

// --- Servo Motor Code ---

#include <Servo.h>

Servo myservo;

void setup() {

myservo.attach(9);  // Attach servo signal pin to digital pin 9

}

void loop() {

for (int pos = 0; pos <= 180; pos += 10) {

myservo.write(pos);

delay(500);

}

for (int pos = 180; pos >= 0; pos -= 10) {

myservo.write(pos);

delay(500);

}

}

| Expt No: 02 | ACTUATOR CONTROL AND PERFORMANCE ANALYSIS |
| --- | --- |
| Date: / /2025 |  |

# AIM:

To control the angular position of a servo motor using Arduino and analyze its performance in terms of accuracy, response time, and stability and rotation of a stepper motor using Arduino and analyze its performance based on step angle, torque, and speed.

# APPARATUS REQUIRED:

  - Microcontroller: Arduino UNO R3,

  - Actuator:

    - Servo Motor (SG90 or MG995),

    - Stepper Motor (28BYJ-48 or NEMA 17).

  - Software: Arduino IDE

  - Components: Jumper Wires, USB-B Cable, Breadboard (optional),

  - Driver Module: ULN2003 or A4988.

# THEORY:

- Servo Motor: This is a closed-loop actuator that provides precise control of angular position. It receives a control signal (PWM) and adjusts its output shaft position accordingly using internal feedback from a potentiometer. The width of the input pulse determines the shaft angle. Pulse width range: Typically, 1ms (0°) to 2ms (180°), Feedback mechanism: Potentiometer inside the motor, Control signal: Generated by Arduino using PWM

- Stepper Motor: This is an electromechanical device that converts electrical pulses into discrete mechanical movements (steps). Each pulse moves the shaft by a fixed step angle, enabling open-loop position control. Stepper motors are commonly used in CNC machines, 3D printers, and precise positioning systems.

# PROCEDURE:

- Servo Motor Control:

- Connections:

      - Servo Motor VCC → Arduino 5V,

      - Servo Motor GND → Arduino GND,

      - Servo Motor Signal → Arduino Digital Pin 9.

- Code: Upload the following sketch to the Arduino.

- Observation: Observe the servo motor shaft rotating from 0° to 180° and back to 0° in smooth steps. The motion should be precise and repeatable according to the PWM signal generated by the Arduino.

// --- Stepper Motor Code ---

#include <Stepper.h>

#define IN1 8

#define IN2 9

#define IN3 10

#define IN4 11

#define stepsPerRevolution 2048 // This value may vary depending on your stepper motor

Stepper myStepper(stepsPerRevolution, IN1, IN2, IN3, IN4);

void setup() {

myStepper.setSpeed(15);

Serial.begin(9600);

Serial.println("Stepper Motor Control");

}

void loop() {

Serial.println("Clockwise");

myStepper.step(stepsPerRevolution);

delay(1000);

Serial.println("Counterclockwise");

myStepper.step(-stepsPerRevolution);

delay(1000);

}

Figure 2: Stepper Motor.

- Stepper Motor Control:

  - Connections:

    - IN1 → Arduino Digital Pin 8,

    - IN2 → Arduino Digital Pin 9,

    - NI3 → Arduino Digital Pin 10,

    - IN4 → Arduino Digital Pin 11,

    - Driver Module VCC → Arduino 5V,

    - Driver Module GND → Arduino GND.

  - Code: Upload the following sketch to the Arduino.

  - Observation: Observe the stepper motor rotating first in the clockwise direction and then in the counterclockwise direction, with smooth stepwise motion corresponding to the programmed number of steps per revolution.

# RESULT AND OBSERVATION:

Both the servo motor and the stepper motor were successfully controlled using the Arduino UNO. The experiments provided a clear, practical understanding of how to manage a servo motor's angular position, noting its accuracy, response time, and stability. Similarly, the control of the stepper motor demonstrated its precise, step-based rotation and the clear relationship between its speed, holding torque, and step angle. Both actuators responded reliably to the programmed commands, confirming the effectiveness of the control methods.