CHAPTER I

INTRODUCTION

Robotics is rapidly transforming modern manufacturing, particularly within assembly and manipulation tasks where speed, repeatability, and precision are paramount. Articulated arms, such as the SCARA (Selective Compliance Assembly Robot Arm) configuration, represent a crucial segment of industrial automation due to their proven efficiency in constrained workspaces. This report details the design, fabrication, and control strategy for a 4-Degree-of-Freedom (4-DOF) SCARA robotic arm prototype, which serves as a practical application of core principles in mechatronics, kinematics, and real-time control systems. The primary intellectual challenge of this work is developing a control system that can execute a planned trajectory accurately, even when built with low-cost components and under strict developmental constraints. This project aims to demonstrate that effective trajectory planning and optimization can successfully bridge the performance gap between industrial-grade and educational-grade robotic hardware, providing a valuable platform for further research in resource-constrained robotics.

  - Scara application overview:

The SCARA configuration is distinguished by its two parallel rotary joints that allow for planar movement with high stiffness in the vertical direction, making it an ideal candidate for swift and repetitive horizontal-plane (pick-and-place) operations commonly found in electronics manufacturing and small-parts assembly. The inherent mechanical compliance in the XY plane, where most of its motion occurs, simplifies precise insertion tasks, distinguishing it from traditional Cartesian or PUMA-style robots. Our prototype is designed to emulate these industrial functionalities, focusing on the core challenge of translating a desired 2D path into synchronized commands for the three primary servo motors using an inverse kinematics model. This focus on the SCARA's fundamental strengths allows the project to concentrate on optimizing the control algorithm to minimize path error & maximize the smoothness of trajectory execution, which is crucial for high-speed assembly processes.

  - Literature Review and Problem Statement

## Contemporary robotics literature consistently emphasizes the dual need for high-precision control and cost-effective implementation, particularly for systems intended for educational platforms or small to medium enterprises (SMEs). The central problem addressed by this project is the successful design and fabrication of a functional SCARA arm capable of reliable trajectory execution, while strictly navigating the resource constraints of a ₹5,000 budget and an accelerated 4-week timeline. This stringent limitation necessitates prioritizing affordable materials like 3D printing filament and economical electronics such as the ESP 8266 and standard servo motors, which presents a significant engineering challenge in achieving industrial-level performance from consumer-grade components. The literature review confirmed that focusing on control system optimization, rather than simply expensive hardware, is the most viable path to achieve the project's ambitious performance goals within the predefined resource envelope, directly addressing the gap between cost and functionality

  - Primary Objectives: Trajectory Control and Kinematic Validation of the Robotics Arm

## The foundation of this project rests on three distinct yet interconnected primary objectives, all focused on achieving precise motion control. The first objective is the systematic design and fabrication of the SCARA's mechanical structure, which must be realized using 3D printing technology for rapid and cost-efficient construction while maintaining structural integrity sufficient for its operational loads. The second, and arguably most critical, goal is the development and validation of a robust forward and inverse kinematics model for the 4-DOF articulated arm, a requirement for accurately translating arbitrary workspace coordinates into the required joint angles. Finally, the ultimate, overarching objective is the successful execution of a pre-programmed, trajectory-based pick-and-place operation, thereby providing conclusive functional proof that the seamlessly integrated mechanical, electronic, and software subsystems can work in concert to achieve the requisite levels of positional and path accuracy.

## CHAPTER II

## METHODOLOGY FOR INCREMENTAL OPTIMIZATION

  - Baseline System Architecture:

## The SCARA system's baseline architecture is strategically divided into three highly interdependent subsystems to manage complexity and facilitate integration. The Mechanical Subsystem comprises the base, the two planar links, and the gripper, all fabricated using 3D printing to meet the budget and timeline constraints. The Electronic Control Subsystem is centralized & selected for its sufficient processing power and ample I/O pins required to drive the three MG995 High-Torque Servo Motors (for the main joints) & a small servo for the gripper. This physical setup is commanded by the Software Subsystem, where the core control logic, including the implementation of the inverse kinematics model and the C++ code for motion sequencing, is written and compiled using the Arduino IDE, establishing a functional, albeit open-loop, starting point for the optimization process.

  - System Evaluation and Benchmarking:

## Initial system evaluation is primarily centered on adhering to the stringent financial constraints, with a total project budget of ₹5,000 being the defining metric. Benchmarking the chosen components, such as the ESP 8266 and the cost-effective servo motors, established a clear performance baseline against which all subsequent optimizations are measured. The "Before" benchmark for positional accuracy is measured using the inherent, open-loop capabilities of the chosen servo motors, documenting their known positional error and potential for overshoot or oscillation. This initial data acquisition provides the crucial foundational reference point necessary for quantifying the performance improvements achieved by the later-implemented software and hardware optimizations, thereby ensuring a verifiable and transparent assessment of the project's overall success in meeting its efficiency goals

  - Optimizing the Control System:

## The optimization phase specifically targets the inherent inaccuracies and limitations of the baseline open-loop servo control system. The key improvement proposed is the integration of sensor feedback, specifically the installation of low-cost rotary encoders on the joints to facilitate the implementation of a more accurate closed-loop control mechanism. The goal is to develop and tune a robust Proportional-Integral-Derivative (PID) controller within the ESP 8266's firmware, which will actively read the actual joint angles from the encoders and minimize the error between the commanded and achieved positions in real-time. This sophisticated control architecture will not only reduce the static positional error and the cumulative path deviation but will also significantly enhance the arm's dynamic performance by damping unwanted oscillations and improving the overall repeatability of the execution, directly addressing the project's core optimization objective.

  - Simplifying and Enhancing Trajectory Planning:

The methodology simplifies trajectory planning by limiting the arm’s operation to the two-dimensional horizontal plane, effectively transforming the complex spatial problem into a manageable planar one, enabling a focus on high-speed execution. The enhancement lies in the sophistication of the path generation, which uses the inverse kinematics model to calculate the joint trajectories, rigorously checking these against three critical factors. These checks include Kinematic Constraints (limits on joint angle, velocity, and jerk to ensure smooth movement), Dynamic Constraints (the torque limits of the servo motors to prevent stalls under light payload), and Workspace Constraints (ensuring the path remains within the arm's reachable area and avoids singularities). This comprehensive planning approach is designed to produce a highly efficient, time-optimized, and dynamically feasible path that maximizes the utilization of the available hardware.

CHAPTER III

SIMULATION AND PERFORMANCE TESTING

  - Simulation Environment:

All mechanical and initial kinematic designs are first rigorously tested within the SolidWorks simulation environment, which serves as the critical validation platform prior to the costly and time-consuming physical fabrication. This environment is used to execute a detailed workspace analysis, confirming the arm's total range of motion and verifying that the links do not encounter any self-collision or joint interference throughout the full range of its operation. Furthermore, the simulation facilitates preliminary Finite Element Analysis (FEA) on the 3D-printed components to assess their structural integrity and stiffness under the expected operational forces, ensuring the mechanical design is sound and reliable before committing to final assembly, thereby reducing project risk and guaranteeing adherence to the 4-week timeline.

  - Simulation Results: Before vs. After

`The primary output of the simulation phase is a quantitative "Before vs. After" comparison focused on validating the kinematic models against the ideal mechanical geometry. The "Before" state represents the theoretical joint angles calculated by the Inverse Kinematics model, defining the intended position in the joint space. The "After" state is established by applying these theoretical joint values to the SolidWorks 3D model and measuring the resultant end-effector position in the simulated workspace. By comparing these two data sets, the team gains high confidence that the implemented mathematical model precisely and accurately reflects the physical geometry of the design, which is a necessary prerequisite for reliable trajectory planning and subsequent control system deployment onto the actual ESP 8266 hardware platform.

CHAPTER IV

EXPERIMENTAL RESULTS AND DISCUSSION

  - Design and Kinematic Validation

The first crucial experimental result is the successful Design Validation, where the physical assembly of the 3D-printed components is confirmed to be stable, rigid, and free of excessive friction in all joints, validating the mechanical integrity developed in the CAD phase. Following this, Kinematic Validation is achieved by running the inverse kinematics code on the ESP 8266 and commanding the arm to execute a series of targeted point-to-point movements across its workspace. The success of this stage is measured by the arm's ability to consistently position its end-effector at the commanded coordinates, proving the microcontroller's ability to translate the calculated joint angles into precise electrical commands and confirming the functional integrity of the entire electromechanical system.

  - Functional Testing of Simple Trajectory Execution

The most important functional test result is the successful execution of a pre-programmed simple trajectory, such as a linear path between two non-adjacent points, which demonstrates coordinated multi-joint movement. This key Week 3 deliverable proves the seamless integration of the software’s trajectory planning logic with the hardware’s motor control, validating the system’s ability to move beyond simple point-to-point positioning into continuous path following. The performance of this trajectory is critically assessed for smoothness, path deviation (error), and execution time, providing tangible metrics that are necessary for the discussion of the system’s overall optimization and its adherence to the project’s goal of achieving precise, coordinated motion control.

  - Discussion of Limitations (Horizontal Plane, Light Loads) and Risk Mitigation

The project’s success must be contextualized by its operational limitations, which are a direct consequence of the imposed ₹10,000 budget and the use of 3D-printed materials. Operation is strictly confined to the horizontal plane and restricted to light payloads, which limits the arm’s general applicability compared to full industrial robots. Addressing potential failures was managed through a comprehensive Risk Mitigation Matrix: Component Delays were counteracted by ordering core P1 components early; Design Flaws were mitigated via mandatory CAD simulation; and the ultimate risk of Budget Overrun was controlled by strictly following the Bill of Materials (BOM) and maintaining a contingency fund. This structured approach ensured that the project maintained its schedule and delivered the functional prototype despite its resource constraints.

CHAPTER V

CONCLUSION AND FUTURE ENHANCEMENT

  - Conclusion:

This project successfully achieved its core objectives by designing, fabricating, and controlling a functional 4-DOF SCARA articulated robotic arm and rigorously validating its ability to execute programmed trajectories. The project's most significant accomplishment was meeting the dual constraints of an 8-week timeline and a ₹10,000 budget by strategically utilizing 3D printing technology and cost-effective Arduino-based control electronics. The successful implementation and validation of the kinematic models, combined with the functional execution of multi-joint motion, confirm the robust integration of the mechanical and control systems. The final prototype serves as a strong, verified platform, demonstrating the efficacy of careful design and software optimization in overcoming the limitations of economical hardware, providing a valuable blueprint for future student projects in constrained robotics environments.

  - Future Work:

## Building upon the validated prototype, future work will focus on three key areas to significantly enhance the arm's performance and utility. The most crucial improvement involves migrating from the current open-loop system to a closed-loop control architecture by integrating low-cost encoders into the joints, which will eliminate positional errors and significantly improve the arm's accuracy and repeatability. Secondly, the functionality will be greatly expanded by developing a Graphical User Interface (GUI), which will allow for intuitive control, real-time data visualization, and the programming of complex, non-linear trajectories. Finally, the long-term goal is to scaling up the design to accommodate increased structural rigidity and a larger workspace, including the incorporation of a dedicated Z-axis mechanism, which would transition the arm toward handling industrial-scale payloads and more complex 3D assembly tasks.