Report on Specifications of Industrial Robots in Robotics Laboratory

Introduction

Robotics plays a vital role in modern automation, enabling precise, repetitive, and high-speed operations across industries. The robotics laboratory provides hands-on exposure to different classes of industrial robots, allowing students to understand their specifications, working principles, and applications. This report summarizes the key specifications and features of three important robots available in the lab: the ABB IRB 1410 Serial Manipulator, the Mitsubishi 6-Axis Industrial Robot, and the ABB IRB 360 FlexPicker Parallel Manipulator.

1. ABB IRB 1410 – Serial Manipulator Robot

- Supplier: ABB, China

- Controller: IRC5

- Payload: 5 kg

- Reach: 1.45 m

- Weight: 225 kg

- No. of Axes: 6

- Types of Motion:

  - Axis 1: Base rotation

  - Axis 2: Arm motion

  - Axis 3: Arm motion

  - Axis 4: Wrist motion

  - Axis 5: Bend motion

  - Axis 6: Turn motion

- Applications: Arc welding, machine tending, and general material handling.

- Remarks: A versatile articulated robot, demonstrating flexibility in motion and the importance of serial link mechanisms.

2. Mitsubishi Electric Industrial Robot (6-Axis)

- Classification: Vertical articulated robot

- Freedom of Movement: 6 axes

- Drive System: AC servo motor

- Position Detection: Absolute encoder

- Payload: 8 kg

- Arm Length: 450 mm + 470 mm

- Maximum Reach Radius: 931 mm

- Body Mass: 41 kg

- Maximum Speed: 10,500 mm/s

- Position Repeatability: ±0.02 mm

- Applications: High-speed pick-and-place, assembly, and inspection.

- Remarks: This robot highlights accuracy and compactness, useful for learning how precision motion and encoder feedback work in automation systems.

3. ABB IRB 360 FlexPicker – Parallel Manipulator Robot

- Supplier: ABB, Bangalore

- Controller: IRC5

- Payload: 3 kg

- Reach: 1.13 m

- Degrees of Freedom: 4

- Integrated Systems:

  - Vision Color Camera: Basler Scout scA1300-32

  - Pick Master Software

  - Color Teach Pendant

  - Conveyor System

- Applications: Packaging, sorting, assembly in food, electronics, and pharmaceutical industries.

- Remarks: A delta-type parallel robot designed for very high-speed pick-and-place operations, illustrating how parallel kinematics differ from serial manipulators.

Conclusion

The three robots studied in the robotics laboratory provide a comprehensive understanding of different robotic architectures and their industrial roles.

- The ABB IRB 1410 demonstrates serial manipulator flexibility and is widely used in welding and material handling.

- The Mitsubishi 6-Axis Robot introduces compact design, servo-driven motion, and precision through encoder-based control.

- The ABB IRB 360 FlexPicker illustrates high-speed automation with vision integration, suited for sorting and packaging tasks.

By comparing these systems, students gain insights into the principles of kinematics, control systems, and industrial applications of robotics, forming a strong foundation for further learning in automation and robotics engineering.