Taguchi Optimization Numerical Problems

- Larger the better

Formula (Larger-the-better S/N)

For a run with n replicates yi:

S/N=-10log⁡10(1ni=1n1yi2)

Problem 1: Three control factors A, B and C each at 3 levels are studied using an L9 orthogonal array. Each run is repeated 3 times. The measured response is a "larger is better" quality characteristic (higher = better). The data:

| Run | A | B | C | Replicates (y) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 65, 67, 66 |
| 2 | 1 | 2 | 2 | 70, 72, 71 |
| 3 | 1 | 3 | 3 | 75, 74, 76 |
| 4 | 2 | 1 | 2 | 68, 69, 70 |
| 5 | 2 | 2 | 3 | 73, 75, 74 |
| 6 | 2 | 3 | 1 | 64, 63, 65 |
| 7 | 3 | 1 | 3 | 78, 77, 79 |
| 8 | 3 | 2 | 1 | 66, 68, 67 |
| 9 | 3 | 3 | 2 | 71, 70, 72 |

Using Taguchi (larger-the-better) S/N ratio, find:

- S/N ratio for each run,

- Factor-level S/N means,

- ANOVA (on S/N) → % contribution,

- Optimal levels,

- Predicted S/N for the optimal combination and an approximate predicted response.

Solution:

1) S/N ratios (per run) — computed values

(rounded to 4 decimal places)

- Run1: 36.3889

- Run2: 37.0234

- Run3: 37.4997

- Run4: 36.7752

- Run5: 37.3830

- Run6: 36.1215

- Run7: 37.8405

- Run8: 36.5196

- Run9: 37.0234

Grand mean of all S/Ns = 36.9528

2) Average S/N by factor level

Factor A (levels 1,2,3):

- A1 mean S/N = 36.9707

- A2 mean S/N = 36.7599

- A3 mean S/N = 37.1278

Factor B (levels 1,2,3):

- B1 mean S/N = 37.0015

- B2 mean S/N = 36.9754

- B3 mean S/N = 36.8815

Factor C (levels 1,2,3):

- C1 mean S/N = 36.3433

- C2 mean S/N = 36.9407

- C3 mean S/N = 37.5744

3) ANOVA on S/N (contribution of factors)

We compute sum of squares using level means and S/N values.

- Total SS (S/N): 2.5105

- SS_A = 0.2045 (df = 2) → MS_A = 0.1023 → F_A = 25.3572

- SS_B = 0.0239 (df = 2) → MS_B = 0.0120 → F_B = 2.9608

- SS_C = 2.2740 (df = 2) → MS_C = 1.1370 → F_C = 281.9782

- SS_Error = 0.0081 (df = 0? Actually df_error = total df − sum factor dfs = 8−6 = 2; MS_error small)

Percent contribution (approx):

- A: 8.15%

- B: 0.95%

- C: 90.58%

Interpretation: Factor C is overwhelmingly dominant for S/N (thus for performance), A has a small effect, B is negligible.

4) Optimal levels (highest average S/N)

- A* = level 3 (A3)

- B* = level 1 (B1) — though B effect is small

- C* = level 3 (C3)

So the best combination by Taguchi analysis is A=3, B=1, C=3.

5) Predicted S/N for the optimal combination & approximate predicted response

Calculate level effects: effect = (level mean − grand mean). Predicted S/N = grand mean + sum (effects of chosen levels).

- Predicted S/N ≈ 37.7981

To convert S/N back approximately to a single expected response y(assume single measurement, n=1 so for larger-the-better S/N ≈ 20·log₁₀(y)):

y≈10(pred S/N)/20

- Predicted (approx) response ≈ 77.61

Problem 2: Three factors A, B and C each at 2 levels. Full factorial 2³ (8 runs). Each run repeated 2 times. The response is larger-the-better. Data:

| Run | A | B | C | Replicates (y) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 120, 118 |
| 2 | 1 | 1 | 2 | 125, 127 |
| 3 | 1 | 2 | 1 | 130, 129 |
| 4 | 1 | 2 | 2 | 134, 135 |
| 5 | 2 | 1 | 1 | 122, 121 |
| 6 | 2 | 1 | 2 | 128, 126 |
| 7 | 2 | 2 | 1 | 135, 136 |
| 8 | 2 | 2 | 2 | 140, 139 |

Same as Problem 1: compute S/Ns, level means, ANOVA on S/N, pick optimal levels, predicted S/N, approximate predicted response.

Solution:

1) S/N ratios (per run), larger-the-better (2 replicates each)

(rounded to 4 decimals)

- Run1: 41.5100

- Run2: 42.0066

- Run3: 42.2452

- Run4: 42.5743

- Run5: 41.6913

- Run6: 42.0753

- Run7: 42.6386

- Run8: 42.8913

Grand mean S/N = 42.2041

2) Average S/N by factor level

Factor A:

- A1 mean = 42.0840

- A2 mean = 42.3241

Factor B:

- B1 mean = 41.8208

- B2 mean = 42.5873

Factor C:

- C1 mean = 42.0213

- C2 mean = 42.3869

3) ANOVA on S/N

- Total SS (S/N): 1.6001

- SS_A = 0.1153 (df = 1) → F_A ≈ 10.9052

- SS_B = 1.1752 (df = 1) → F_B ≈ 111.1514

- SS_C = 0.2673 (df = 1) → F_C ≈ 25.2806

- SS_Error = 0.0423

Percent contributions:

- A: 7.21%

- B: 73.45%

- C: 16.70%

Interpretation: Factor B is the major contributor to the S/N (thus to performance) in this dataset; A and C have smaller effects.

4) Optimal levels (highest average S/N)

- A* = level 2

- B* = level 2

- C* = level 2

Best combination: A=2, B=2, C=2.

5) Predicted S/N & approximate predicted response

- Predicted S/N ≈ 42.8902

- Approximate predicted response (single measurement conversion)
y≈10(pred S/N)/20≈139.48

Recommendation: perform confirmation runs at A=2, B=2, C=2 and check mean response and S/N to validate prediction.

- Smaller the better

Formula (smaller-the-better S/N)

For a run with n replicates yi:

η  =  -10log⁡10(1ni=1nyi2)

Higher η (less negative) ⇒ better (smaller and/or less variable y).

Problem 1: L9 (3 factors × 3 levels), 3 replicates (smaller-the-better). Data:

| Run | A | B | C | Replicates y |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 4.5, 4.7, 4.6 |
| 2 | 1 | 2 | 2 | 3.9, 4.0, 4.1 |
| 3 | 1 | 3 | 3 | 3.2, 3.1, 3.3 |
| 4 | 2 | 1 | 2 | 4.2, 4.1, 4.3 |
| 5 | 2 | 2 | 3 | 3.5, 3.6, 3.4 |
| 6 | 2 | 3 | 1 | 4.8, 4.9, 4.7 |
| 7 | 3 | 1 | 3 | 2.9, 3.0, 2.8 |
| 8 | 3 | 2 | 1 | 4.6, 4.5, 4.4 |
| 9 | 3 | 3 | 2 | 3.7, 3.8, 3.6 |

Solution:

1) S/N per run (smaller-the-better),

Compute η=-10log⁡10(13∑yi2).

- Run1: −13.2565

- Run2: −12.0430

- Run3: −10.1058

- Run4: −12.4666

- Run5: −10.8837

- Run6: −13.6261

- Run7: −9.2514

- Run8: −13.0657

- Run9: −11.3661

Grand mean S/N (all runs): −11.7850

2) Level means (m-values)

Average S/N for runs where factor = that level:

- Factor A:
mA1 = −11.8018, mA2 = −12.3255, mA3 = −11.2277

- Factor B:
mB1 = −11.6582, mB2 = −11.9975, mB3 = −11.6994

- Factor C:
mC1 = −13.3161, mC2 = −11.9586, mC3 = −10.0803

(Higher m = better)

3) ANOVA on S/N

Compute sum-of-squares (S/S) for each factor from level means vs grand mean.

- SS_A = 1.8088

- SS_B = 0.2057

- SS_C = 15.8410

- SS_Total (S/N) = 17.9528

- SS_Error ≈ 0.0973

Percent contributions:

- A: (1.8088 / 17.9528) = 8.15%

- B: 1.15% (≈0.206 / 17.953)

- C: 88.2% (≈15.841 / 17.953)

Interpretation: Factor C dominates; A has a small effect; B is negligible.

4) Optimal levels (choose level with highest m)

- A* = level 3 (mA3 = −11.2277)

- B* = level 1 (mB1 = −11.6582)

- C* = level 3 (mC3 = −10.0803)

Optimal combination: A=3, B=1, C=3.

5) Predicted S/N and approximate predicted response

Predicted S/N for the optimal combination:

ηpred=grand mean+(mA3-gm)+(mB1-gm)+(mC3-gm)

Numerically: ηpred≈-9.3962.

Convert to approximate expected response (single measurement approximation for smaller-the-better):
From η=-20log⁡10(y)⇒ y≈10-η/20.

Predicted response ≈ 2.9499 (units same as y).

Problem 2: Full factorial 23, 2 replicates (smaller-the-better). Data:

| Run | A | B | C | Replicates y |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 5.2, 5.1 |
| 2 | 1 | 1 | 2 | 4.8, 4.9 |
| 3 | 1 | 2 | 1 | 4.5, 4.6 |
| 4 | 1 | 2 | 2 | 4.0, 4.1 |
| 5 | 2 | 1 | 1 | 5.0, 4.9 |
| 6 | 2 | 1 | 2 | 4.6, 4.7 |
| 7 | 2 | 2 | 1 | 4.2, 4.3 |
| 8 | 2 | 2 | 2 | 3.8, 3.9 |

Solution:

1) S/N per run (smaller-the-better),

- Run1: −14.2366

- Run2: −13.7153

- Run3: −13.1608

- Run4: −12.1498

- Run5: −13.8925

- Run6: −13.3496

- Run7: −12.5684

- Run8: −11.7099

Grand mean S/N = −13.09785

2) Level means

- Factor A: mA1 = −13.3156, mA2 = −12.8801

- Factor B: mB1 = −13.7985, mB2 = −12.3972

- Factor C: mC1 = −13.4646, mC2 = −12.7311

3) ANOVA on S/N — quick summary

- SS_A = 0.3793

- SS_B = 3.9272

- SS_C = 1.0758

- SS_Total = 5.4822

- SS_Error ≈ 0.1000

Percent contributions:

- A: 6.92%

- B: 71.63%

- C: 19.62%

Interpretation: Factor B is the dominant contributor (largest effect), C moderate, A small.

4) Optimal levels (highest m)

- A* = level 2 (mA2 = −12.8801)

- B* = level 2 (mB2 = −12.3972)

- C* = level 2 (mC2 = −12.7311)

Optimal combination: A=2, B=2, C=2.

5) Predicted S/N and approximate predicted response

Predicted S/N ≈ −11.8128.

Approximate predicted response:
y≈10-η/20⇒y≈3.8962.

- Nominal the best

S/N for Nominal-the-best

For replicates yi(with mean yand sample variance s2):

η  =  10log⁡10(y2s2)

Problem 1: L₉ (3 factors A,B,C each with 3 levels), 3 replicates per run. Response should be as close as possible to a target (nominal-is-best). Data:

- A1 B1 C1 : 50.2, 49.8, 50.5

- A1 B2 C2 : 49.7, 50.1, 49.9

- A1 B3 C3 : 50.8, 50.6, 50.9

- A2 B1 C2 : 49.3, 49.5, 49.4

- A2 B2 C3 : 50.0, 49.8, 50.1

- A2 B3 C1 : 51.2, 51.0, 51.1

- A3 B1 C3 : 50.5, 50.4, 50.6

- A3 B2 C1 : 49.9, 50.2, 50.0

- A3 B3 C2 : 50.7, 50.8, 50.6

Solution:

- S/N (nominal) per run

(Using yand sample variance s2. Values rounded)

| Run | Mean y | s2 | S/N η=10log⁡10(y2/s2)(dB) |
| --- | --- | --- | --- |
| 1 | 50.1667 | 0.123333 | 43.0975 |
| 2 | 49.9000 | 0.040000 | 47.9414 |
| 3 | 50.7667 | 0.023333 | 50.4318 |
| 4 | 49.4000 | 0.010000 | 53.8745 |
| 5 | 49.9667 | 0.023333 | 50.2938 |
| 6 | 51.1000 | 0.010000 | 54.1684 |
| 7 | 50.5000 | 0.010000 | 54.0658 |
| 8 | 50.0333 | 0.023333 | 50.3054 |
| 9 | 50.7000 | 0.010000 | 54.1002 |

Grand mean of S/N (all runs): 50.9199 dB

- Level means of S/N (m-values)

Average S/N for runs where factor at that level:

- Factor A:

  - mA1 = 47.1569

  - mA2 = 52.7789

  - mA3 = 52.8238

- Factor B:

  - mB1 = 50.3459

  - mB2 = 49.5135

  - mB3 = 52.9001

- Factor C:

  - mC1 = 49.1904

  - mC2 = 51.9720

  - mC3 = 51.5971

(For nominal-is-best, higher η is better.)

- Quick ANOVA on S/N (percent contributions)

Compute sum of squares of S/N and partition to factors. (Each level appears 3 runs in L₉.)

- Total SS (S/N) ≈ 110.363

- SS_A ≈ 63.723 → 57.74%

- SS_B ≈ 18.686 → 16.93%

- SS_C ≈ 13.670 → 12.39%

- SS_Error ≈ 14.284 (remaining)

Interpretation: Factor A is the largest contributor to variation in S/N, B and C smaller.

- Choose optimal levels (highest m)

- A*: level 3 (mA3 = 52.8238)

- B*: level 3 (mB3 = 52.9001)

- C*: level 2 (mC2 = 51.9720)

⇒ Predicted best combination: A=3, B=3, C=2.

5) Predicted S/N and predicted mean (Taguchi prediction)

Predicted S/N (additive on S/N means):

ηpred=grand mean(S/N)+(mA3-gm)+(mB3-gm)+(mC2-gm)

Numerically: η_pred ≈ 55.8562 dB.

Predicted mean response (using additive model on run means):

Run means (per run) used to form level means of y. Predicted mean at (A3,B3,C2) ≈ 50.7037.

Problem 2 — Full factorial 23(A,B,C each at 2 levels), 2 replicates per run. Data:

- A1 B1 C1 : 100.2, 99.8

- A1 B1 C2 : 100.5, 100.1

- A1 B2 C1 : 99.7, 99.9

- A1 B2 C2 : 100.0, 100.2

- A2 B1 C1 : 100.8, 100.6

- A2 B1 C2 : 101.0, 100.9

- A2 B2 C1 : 99.5, 99.4

- A2 B2 C2 : 99.8, 99.6

Solution:

- S/N (nominal) per run (rounded)

| Run | Mean y | s2 | S/N η(dB) |
| --- | --- | --- | --- |
| 1 | 100.0000 | 0.080000 | 50.9691 |
| 2 | 100.3000 | 0.080000 | 50.9951 |
| 3 | 99.8000 | 0.020000 | 56.9723 |
| 4 | 100.1000 | 0.020000 | 56.9984 |
| 5 | 100.7000 | 0.020000 | 57.0503 |
| 6 | 100.9500 | 0.005000 | 63.0924 |
| 7 | 99.4500 | 0.005000 | 62.9624 |
| 8 | 99.7000 | 0.020000 | 56.9636 |

Grand mean S/N ≈ 57.0005 dB

- Level means of S/N

Each level appears 4 runs in a 23design.

- Factor A: mA1 = 53.9837, mA2 = 60.0172

- Factor B: mB1 = 55.5267, mB2 = 58.4742

- Factor C: mC1 = 56.9885, mC2 = 57.0124

- ANOVA on S/N (percent contributions)

Total SS (S/N) ≈ 145.10

- SS_A ≈ 72.805 → 50.17%

- SS_B ≈ 17.375 → 11.97%

- SS_C ≈ 0.0011 → ~0.00%

- SS_Error ≈ 54.922 (remaining)

Interpretation: Factor A is the dominant contributor to S/N; B moderate; C negligible.

- Optimal levels (highest m)

- A* = level 2 (mA2 = 60.0172)

- B* = level 2 (mB2 = 58.4742)

- C* = level 2 (mC2 = 57.0124)

⇒ Predicted best combination: A=2, B=2, C=2.

- Predicted S/N and predicted mean

- Predicted S/N (additive on S/N means): η_pred ≈ 61.5028 dB.

- Predicted mean response (from run means level-effects): ≈ 99.9750.