6 AXIS Industrial Robot Mitsubishi Electric (RV-8CRL-D)

A vertical multi-joint robot (8 kg payload, 931 mm reach) with a CR800 D controller and an R32TB teach pendant, equipped with dual/three-jaw grippers.

Figure 1. Mitsubishi 6-Axis Industrial Robot Joint Diagram

Specifications:

- Industrial Robot: Mitsubishi Electric RV-8CRL

- Robot Type: Vertical articulated robot

- Degrees of Freedom: 6 axes

- Drive System: AC servo motor

- Position Detection: Absolute encoder

- Maximum Payload Capacity: 8 kg

- Robot End of Arm Tooling (EOAT): As Per Application (Dual two-Jaw and Three-Jaw gripper)

- Controls Panel: FX-5U PLC Control Panel & HMI Operator Panel

- Software: Robot Simulation Software for 01 User

Applications:

- Pick & Place

- Machine Loading & Unloading

- Palatizing,

- Conveyor Tracking, Conveyor Logics, Conveyor Pick & Place

- Robot Path Teaching Drawing

- Practical Board for glueing path, Circular path programming, SPLINE Programming

- Robot TCP Calculation, HMI screen developing for controlling Robot operations, PLC master logic programming, Robot vs human Door & Emergency safety Interlock system. Dual EOAT for the loading and unloading process.

AI-Based 6-Axis Mitsubishi Collaborative Robot (Cobot)

Configuration:

- 6-Axis AI-enabled Collaborative Robot

- Payload: 5 kg

- Reach: 900 mm radius

- Position Repeatability: ±0.03 mm

- High-Speed Mode: 1000 mm/s

- Collaborative Modes: 250 mm/s and 50 mm/s

- Includes Controller and Teach Pendant (7 m cable)

Safety & Interface Features:

- Multi-color LED status indicator

- Standard Digital Inputs/Outputs (DIs/DOs) and Safety I/Os

- Ethernet compatibility

- Plug-and-play interface for electric gripper, force sensor, and vision system

- Built-in solenoid valve set for pneumatic gripper actuation

- Integrated air piping and pneumatic tubing

Applications

End Effectors

- Electric Sensorized Gripper (1 kg payload)

  - Strain gauges, flexi-force sensor, IR sensor, load cell

  - PLC-controlled

  - Parallel jaw and curvilinear jaw options

- Two-Finger Pneumatic Gripper (500 g payload)

- Magnetic Gripper (500 g payload)

- Vacuum Gripper (500 g payload)